# Project Title:

# IoT & Computer Vision-Based Smart Stick for Visually Impaired person

Project Components:

1. ESP32-CAM

2. ESP32

3. Df Player Mini

4. SD Card

5. Speaker

6. Power Bank 10000mah

7. ON/OFF Button

8. 470 Micro Farad two capacitor and one is 47microfarad

9. Jumper Wires

10. Type B Micro USB cable

11. Vero Board

12. Double Sided tape

13. Glue gun

14. Soldering material

15. Box for hardware fix on stick

16. One pvc pipe with based of wood

17. Laptop for esp32 cam video processing

18. Ultrasonic sensor

19. Buzzer

Project Working :

- Initially one the button power both controller esp32 cam and esp32

- On the mobile hotspot

- Check the three devices laptop esp32, esp32 cam connected with same network.

- First check the ip put into the chrome http protocol it should show the stream.

- Second ENB button of the esp32 push then two voices play during power 001.mp3 and 002.mp3.

- Third is Laptop code these control url and ip put OpenCV YOLO V8n model use it process first the frame and detection result send to the esp32 it handle this result it will connect df player mini play the voices first two was initial but 003.mp3 to 082.mp3 is a coco 80 classes pre-recorded voices record into the sd card. Known object it play the voices but if unknown object detect in frame then it will be the 083.mp3.

- All these working untill mobile hotspot is one. This was all online mode but it goes offline for 5 second than ultrasonic sensor work start measure the distance and secondary output with buzzer beap play 1.5 second.

Component Connection:

ESP32-CAM:

It was directly connect with main source power bank but vcc(5V) and gnd we connect the 470 micro farad capacitor.

ESP32:

- Df player mini connect with ESP32 UART 2 pin 16 and 17 pin.

- Through the df player mini 1 kilo ohm resistor add Rx to Tx2 of esp32

- Df tx connect with Rx2 of esp32.

- Power pins of df player mini  vcc connect with the esp32 3.3v and gnd to gnd but before esp32 3.3v and gnd connect with df player mini add the 470 micro farad capacitor

- Speaker connect with the spk1 pin of the df player mini connect with the positive an sp2 with  gnd.

- Ultrasonic connect with the esp32 pin 5 trig and pin 18 connect with the echo but make the voltage divider for protection

- Vcc and gnd of ultrasonic sensor connect with the 47 micro farad capacitor directly connect with esp32p pin 3.3 v and gnd.

- Buzzer attach with the esp32 pin 4 with buzzer pin vcc mean positive and gnd to gnd.

- Led pin is 2 but it builtin  led on during detection result receive the esp32 through the laptop.

- If add the led with pin 2 of esp32 its okay..

Location of Components: